MSU-2  •  EXPLORING SYSTEM-ON-CHIP ARCHITECTURES

Session Recap — August 4, 2026

ZDMA branch closed with register proof  •  Round-6 CDMA pivot opened and de-risked in simulation

| Project | MSU-2, Task 2 — OS vs. Bare-Metal SPI Characterization (ZCU102 / Zynq UltraScale+ MPSoC) |
| --- | --- |
| Lab | Cybersecurity Assurance & Policy (CAP) Center, Morgan State University |
| Repository | jbc0510/spi-soc-research, branch feature/dma-benchmarking |
| Session commits | aa92da8 → 26b9642 → 0fe00c1 → f3958cb → 9eaa6bb → 3c8776c → ad6c3c4 |
| Machines | stile (Vivado/Vitis 2025.1, builds, sim); jeremiahc (SD flash); ZCU102 (3 boots) |
| Date | Tuesday, August 4, 2026 |

The one-line summary of this session The five-round ZDMA investigation was definitively closed — hardware register readback proved our fixed-address configuration landed in silicon while the failure persisted unchanged, and AMD’s own documentation confirms the PS DMA engine cannot service a peripheral FIFO. The same afternoon, the round-6 pivot (PL-side AXI CDMA with KeyHole mode) was planned, committed, and validated in simulation: the CDMA’s fixed-address bursts were observed working on the bus, and a SmartConnect interconnect limitation that would have failed on hardware was caught pre-silicon. The July MSR was also built and finalized.

# Contents

# 1. Where this session sits in the project

The contract asks how much an operating system costs SPI communication, and whether DMA can beat processor-driven transfers (SOW task 2.b). The PIO answer is complete (fixed ~28 µs OS toll; 3–140× determinism advantage for bare-metal). The DMA arm had been blocked for five rounds by a persistent data-corruption signature on the PS-side ZDMA engine. This session ended that investigation with a definitive answer and opened the supported alternative.

# 2. Round 5 — the ZDMA branch, closed for good

## 2.1 The experiment

Candidate #1 from the round-4 handoff: set DstBurstLen=0 (TX) and SrcBurstLen=0 (RX) so every AXI beat is a standalone fixed-address transaction, plus a DATA_ATTR register readback printed on UART after each configuration call — so the hardware itself testifies whether the settings took.

Full pipeline on stile: edit → Vitis build → bootgen → SD → boot → UART capture, with the md5 chain verified at every hop: source 8039e01b → ELF 911e3af1 → BOOT cb9548d5 → card.

## 2.2 The verdict — three results

| # | Result | Meaning |
| --- | --- | --- |
| 1 | Config provably landed: TX DATA_ATTR 0x0483C200, RX 0x0080120F — FIXED+LEN0 confirmed in silicon, stable across 3 boots | “Driver dropped the config” is eliminated forever |
| 2 | Stride unchanged: correct bytes = payload/256 at every size — identical to round 4 | The address walk is the engine’s internal counter, not AXI burst bookkeeping; simple mode has no freeze knob |
| 3 | physics_fail=11: 64 KiB “completed” in ~474 µs (~138 MB/s) vs ~0.54 s wire floor | Zero flow control between ZDMA and the SPI FIFO — the pairing cannot work even with perfect addressing |

External confirmation (AMD documentation, retrieved this session): the ZynqMP DMA standalone-driver wiki states peripheral DMA is not tested/supported; the ZDMA Linux wiki notes there is no flow-control mode; Xilinx staff confirm FIXED bursts exist in simple mode only, ruling out scatter-gather as a fallback. The round-4 physics_fail count was also confirmed as 11 from the round-4 anchor doc — the no-pacing behavior is an invariant of the pairing, present before this session’s change.

The finding, stated for the record ZDMA (LPD-DMA) + PS SPI1 is an architecturally unsupported pairing on ZynqMP: the PS SPI controller exposes no DMA request interface, and the ZDMA is a memory-to-memory engine with no flow control and no hold-address mode. This is a documented negative result directly serving SOW task 2.b — true DMA-driven SPI on this platform requires the QSPI controller’s built-in DMA or a PL-side DMA engine.

# 3. Sponsor reporting

- July MSR built, corrected, and finalized (MSR_MSU_Project_2_July2026.docx): two risks (Vitis toolchain — mitigated via stile; PS-DMA feasibility — realized and retired), eight Task-2 progress items led by the completed Linux jitter recapture. DMA silicon-run dating counted for July per PI decision. Due to Booz Allen by August 5.

- Interim Progress Report status confirmed: delivered to David Lattimore, dated July 31. Every open item in it (jitter recapture, ZDMA validation, Vitis blockage) has since resolved favorably — strong opening material for the August cycle.

# 4. Round 6 — the CDMA pivot, opened and de-risked

## 4.1 Gate checks and plan

- Vivado synthesis license for xczu9eg confirmed working on stile (one-LUT probe synth, 25 s) — the May-report license risk is retired. Round 6 has no external blockers.

- Design direction verified against AMD PG034: AXI CDMA (memory-to-memory) has a documented KeyHole Write/Read mode — fixed-address AXI transactions — exactly the capability ZDMA lacks. Constraint: Max Burst Length = 16 when keyhole is on. NOT AXI DMA (that IP is MM↔Stream; Quad SPI has no stream port — the same interface-class mismatch that sank ZDMA). Plan committed as dma/ROUND6_PLAN.md.

## 4.2 Block-design delta (committed, validated)

hardware/add_cdma_r6.tcl — a 60-line delta sourced after the untouched canonical BD Tcl (write_bd_tcl round-trip deliberately avoided: the EMIO net overrides do not survive regeneration). Adds axi_cdma_0 (simple mode, 32-bit, MaxBurst 16), enables PS S_AXI_HP0_FPD for the DDR path, grows SmartConnect to 2 SI / 3 MI, assigns the four address segments, and explicitly excludes the fan-out artifacts. Validated clean: zero unassigned-segment warnings, four segments verified.

## 4.3 The keyhole simulation campaign — five runs, three findings

Rather than build a bitstream and debug on silicon (the approach that cost five hardware rounds on ZDMA), a sim-only block design was built: AXI VIP master (CPU stand-in) + real axi_cdma + real axi_quad_spi + VIP slave memory (DDR stand-in), same addresses as the real design, with a testbench running a keyhole transfer AND an INCR negative control. Five iterations in ~90 minutes, converging via anchor-value reads (QSPI SPICR = 0x180 proved the read path) and bus-level $display monitors on the netlist wires.

| Finding | Evidence | Consequence |
| --- | --- | --- |
| CDMA keyhole works | Bus monitors: AWBURST=FIXED at 0xA0000068 held within and across bursts (len=5 then len=9, both at the DTR) | The round-6 core mechanism is verified at RTL level — PG034 behavior confirmed |
| SmartConnect blocker (caught pre-silicon) | Phase-A FIXED writes never reached the QSPI (no M00 traffic); CDMA got DECERR (CDMASR=0x5042) | SmartConnect will not convert multi-beat FIXED bursts to AXI4-Lite. The committed delta, as wired, would fail identically on hardware. Fix identified: QSPI full-AXI4 slave mode (verify in PG153/PG247) |
| INCR negative control — textbook capture | The INCR transfer walked 0x68,6C,70,74,78,7C then wrapped through the 7-bit Lite window into 0x00,04,08… — clobbering QSPI control registers | Publication-grade illustration of why incrementing DMA into register space is catastrophic |

Why this matters methodologically The same fixed-address question was answered three independent ways in one day: hardware registers on silicon (round 5), vendor documentation, and bus-level waveforms in simulation (round 6). The simulator caught in ninety minutes a design bug that hardware iteration would have cost days — and produced the positive result (keyhole FIXED bursts observed) that the entire pivot depends on, before any bitstream exists.

## 4.4 Block-diagram exports

Vivado renders of both topologies committed to hardware/docs/ (PDF + SVG each): bd_spi_benchmark_original and bd_spi_benchmark_cdma_r6. Export script at scripts/export_bd_diagrams.tcl. Tool gotcha recorded: in 2025.1, write_bd_layout requires a GUI context for ALL formats — run via vivado -mode gui -source (or xvfb-run, now installed on stile); batch mode fails with BD 5-349. The cdma_r6 render shows the PRE-FIX SmartConnect wiring — re-export after the fix so the paper figure matches the validated design.

# 5. Hard-won lessons (add to the checklist)

- Keep pasted heredoc blocks under ~3 KB (split cat > / cat >>). A >4 KB paste garbled the terminal display mid-session — content survived, but it cost a diagnosis cycle.

- Start screen -L -Logfile <path> BEFORE powering the board. Round 5’s UART log had to be recovered from scrollback hardcopy.

- Marker lines must gate on command results. The BD-export script printed “done” over failed exports; the real gate is the output files existing.

- Anchor-first testbenches: read a known reset value (QSPI SPICR = 0x180) before trusting any readback — the sim-domain version of the decimal/hex lesson.

- AXI VIP read API needs a wide (4096-bit) output vector; a 32-bit variable silently receives zeros. Writes are unaffected — the asymmetry masqueraded as a hardware fault for two runs.

- Vitis ws2 keeps its own SOURCE COPY of the benchmark — the repo-to-ws2 cp before build is load-bearing. A symlink cleanup would retire this bug class.

# 6. Where everything lives

| Artifact | Path (repo, feature/dma-benchmarking) |
| --- | --- |
| Round-5 anchor result (the closure record) | dma/ANCHOR_RESULT_round5.md |
| Round-5 UART evidence | dma/logs/uart_round5_burstlen0.log |
| Round-6 plan | dma/ROUND6_PLAN.md |
| CDMA block-design delta | hardware/add_cdma_r6.tcl |
| Keyhole sim (BD, testbench v4, runner, findings) | hardware/sim/ — sim_keyhole_bd.tcl, tb_keyhole.sv, run_keyhole_sim.tcl, R6_SIM_FINDINGS.md |
| Sim bus-truth log (run 5) | dma/logs/r6_keyhole_sim5_bus_truth.log |
| BD renders + export script | hardware/docs/bd_spi_benchmark_*.{pdf,svg} • scripts/export_bd_diagrams.tcl |
| July MSR (send to Booz Allen) | MSR_MSU_Project_2_July2026.docx (local, not in repo) |

# 7. Next-session runbook

- Read PG153 (Quad SPI C_TYPE_OF_AXI4_INTERFACE / full-AXI4 slave constraints) and PG247 (SmartConnect FIXED-burst support) — this decides the exact fix.

- Revise add_cdma_r6.tcl (likely: QSPI to full-AXI4 slave so FIXED bursts pass unconverted). Re-run run_keyhole_sim.tcl as the gate — PASS means M00 shows 16 writes at 0x68 and OCY=15.

- Switch the testbench verdict to the bus monitors (and fix the VIP read plumbing) so PASS/FAIL is computed from ground truth.

- Only after sim PASS: bitstream via scripts/rebuild_bitstream.tcl (+ delta), export XSA, rebuild the ws2 platform, port the benchmark to CDMA registers (keyhole = CDMACR bits 4/5 at runtime, chunk-paced ≤ 256 B). Re-export the BD render.

- Housekeeping: .gitignore for vivado.log/jou and *.pb; send the July MSR if not yet sent; queue the negative-result + keyhole slide for the biweekly deck.

State of the branch at session end Nothing dangling. Rounds 1–5 closed with zero open questions; round 6 steps 1–2 committed, step 3 (sim) has produced its central positive result and one precisely-characterized blocker with an identified fix. No external blockers: license, tools, and pipeline all verified. The next session starts with two datasheet pages and a one-file edit, gated by an existing simulation.